Confined Space Entry & Atmospheric Monitoring Procedure — "Permit-Required" Spaces

ORG-NTD-000010-HSE-PRC-00022

| Revision History |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rev | Date | Revision Description | Originator | Reviewer/ Endorser | Approver |
| 0 | 01-15-25 | Issued for Review | M. O'Brien | Director, HSSE | VP Operations |
| A | 04-16-26 | Revised per lessons learned — added §5 atmospheric monitoring & rescue plan requirements | M. O'Brien | Sr. Dir., HSSE | SVP Operations |

Change Log

| Revision | Description of Changes and Notes |
| --- | --- |
| 0 | Initial issue — internal review only. |
| A | Added atmospheric monitoring thresholds (O₂, LEL, H₂S, CO). Updated rescue plan to reference Rio Grande LNG (RGLNG) site-specific requirements. Incorporated OSHA 1910.146 & API RP 2016 cross-references. |

Table of Contents

1.0	Purpose	4

2.0	Scope	4

3.0	Roles and Responsibilities	4

4.0	Safety and Health Precautions	4

4.1	Personal Protective Equipment	4

4.2	Chemicals and Hazards	4

4.3	Environmental Requirements	5

5.0	Procedure Title (Enter procedure here.)	5

6.0	Record Keeping Requirements	5

7.0	Training	5

8.0	Sample Styles	6

8.1	Bullets	6

8.2	List Number	6

8.3	Tables	7

8.4	Figures	7

9.0	Definitions	7

9.1	Terms	7

9.2	Abbreviations and Acronyms	8

10.0	References	8

Appendix A.	Example 1	9

Appendix B.	Example 2	10

Purpose

The purpose of this procedure is to establish requirements for the safe entry into and work within permit-required confined spaces — including vessels, tanks, sumps, silos, and enclosed pipework — at facilities owned or operated by NextDecade Corporation, its subsidiary Rio Grande LNG (RGLNG), or any affiliated entity. "Permit-required" means the space has one or more hazards as defined in 29 CFR 1910.146(b).

Scope

This procedure applies to all confined space entry (CSE) activities at any NextDecade or RGLNG facility. It covers employees, contractors, and any other members of the Workforce (as defined in §9.1) who authorize, enter, attend, rescue, or supervise CSE operations. Exclusions: atmospheric-only hazards in spaces <4 feet deep that can be made safe by continuous forced-air ventilation alone may use the short-form Alternative Entry procedure (Appendix C).

Roles and Responsibilities

Roles and responsibilities for this procedure include the following.

| Role | Responsibilities |
| --- | --- |
| Entry Supervisor | Authorize entry by signing the CSE Permit after verifying all preconditions. Cancel the Permit immediately on any stop-work trigger. Ensure the Attendant and Entrants are trained per ORG-NTD-000010-HSE-PRC-00037. |
| Authorized Entrant | Enter the space only after a valid Permit is posted at the entry point. Maintain continuous communication with the Attendant. Exit immediately on any alarm — atmospheric, personal, or facility. |
| Attendant (Hole Watch) | Maintain an accurate headcount of all Entrants in the space at all times. Monitor atmospheric readings transmitted from the Entrant's 4-gas monitor. Summon rescue resources — never enter the space to attempt rescue. |
| HSSE Advisors | Calibrate & bump-test 4-gas monitors before each shift. Verify the rescue plan is current and rescue equipment is staged per Appendix B. |
| Contractors & Visitors | Comply with this procedure as if directly employed by NextDecade. Contractor CSE Permits require dual sign-off (contractor supervisor + NextDecade Entry Supervisor). |

Safety and Health Precautions

Personal Protective Equipment

All Entrants shall wear: hard hat, safety glasses with side shields, FRC, leather work boots, and a full-body harness with retrieval D-ring attached to a mechanical retrieval device (except where the configuration of the space makes retrieval impractical — document the waiver on the Permit).

Respiratory protection: supplied-air respirator (SAR) or SCBA when atmospheric monitoring shows O₂ < 19.5% or any toxic gas above its PEL/TLV. PAPR is acceptable only for nuisance particulates in oxygen-normal atmospheres.

Attendants shall carry a 20-lb ABC fire extinguisher within arm's reach. Rescue Entrants shall be equipped per the site Rescue Plan (Appendix B).

Confined Space Entry & Atmospheric Monitoring Procedure

The following steps implement the Permit-Required Confined Space Entry process from planning through close-out. All atmospheric readings shall be documented in real-time on the attached Atmospheric Monitoring Log (Appendix A, Section 2).

The Entry Supervisor completes the following:

| Step | Description |
| --- | --- |
| - 1 | Review the Job Safety Analysis (JSA) or Task Risk Assessment (TRA) with all entry team members ≥30 minutes before planned entry. |
| - 2 | Verify energy isolation (lockout/tagout per ORG-NTD-000010-HSE-PRC-00021) and blanking/blinding of all piping connections. |
| - 3 | Direct a pre-entry atmospheric test: O₂ (19.5–23.5%), LEL (<10% of LFL), H₂S (<10 ppm TWA), CO (<25 ppm TWA). Record on the Atmospheric Monitoring Log. |
| - 4 | Post the signed Permit at the entry point — visible to the Attendant. |
| - 5 | Confirm continuous ventilation is operational and the Attendant has line-of-sight or hardwired communication with every Entrant. |
| - 6 | Authorize entry. Entrants sign in on the headcount log. |
| - 7 | Monitor: Attendant reviews real-time atmospheric data every 5 minutes. If any parameter exceeds the action level — evacuate, ventilate, retest. |
| - 8 | Close out: all Entrants sign out; Attendant signs the Permit; Entry Supervisor cancels the Permit. File per ORG-NTD-000010-HSE-PRC-00037. |

Record Keeping Requirements and Training

All CSE Permit records, Atmospheric Monitoring Logs, and rescue drill records shall be retained per ORG-NTD-000010-HSE-PRC-00037 — Manual — Procedures, Definitions and Record Keeping Requirements. Minimum retention: 5 years or as required by applicable regulation, whichever is longer.

Definitions

Terms

The following terms are specific to this document.

| Term | Definition |
| --- | --- |
| Confined Space | A space that (a) is large enough for an employee to bodily enter, (b) has limited or restricted means of entry or exit, and (c) is not designed for continuous occupancy — per 29 CFR 1910.146(b). |
| Permit-Required Confined Space | A confined space that has one or more of the following: hazardous atmosphere, engulfment potential, inwardly converging walls/floor, or any other recognized serious safety or health hazard. |
| Authorized Entrant | An employee who is trained and authorized to enter a permit-required confined space. |
| Attendant (Hole Watch) | A trained individual stationed outside the confined space who monitors Entrants, maintains headcount, and summons rescue — never enters the space. |
| Entry Supervisor | The person responsible for authorizing entry, verifying preconditions, and canceling the Permit upon completion or on any stop-work trigger. |
| Action Level | The atmospheric monitoring threshold at which the Entry Supervisor must evacuate, ventilate, and retest: O₂ <19.5% or >23.5%, LEL ≥10%, H₂S ≥10 ppm, CO ≥25 ppm. |

Abbreviations and Acronyms

The following abbreviations and acronyms are specific to this document.

| Abbreviation/Acronym | Definition |
| --- | --- |
| CSE | Confined Space Entry |
| JSA | Job Safety Analysis |
| TRA | Task Risk Assessment |
| LEL | Lower Explosive Limit |
| LFL | Lower Flammable Limit |
| PEL | Permissible Exposure Limit |
| TLV | Threshold Limit Value |
| TWA | Time-Weighted Average |
| SAR | Supplied-Air Respirator |
| SCBA | Self-Contained Breathing Apparatus |
| PAPR | Powered Air-Purifying Respirator |
| OSHA | Occupational Safety and Health Administration |
| RGLNG | Rio Grande LNG |
| PPE | Personal Protective Equipment |
| FRC | Flame-Resistant Clothing |

References

| Document Number | Document Title |
| --- | --- |
| ORG-NTD-000010-HSE-STD-00001 | NextDecade HSSE Standard |
| ORG-NTD-000010-HSE-PRC-00021 | Lockout/Tagout Procedure |
| ORG-NTD-000010-HSE-PRC-00037 | Manual — Procedures, Definitions and Record Keeping Requirements |
| External | OSHA 29 CFR 1910.146 — Permit-Required Confined Spaces |
| External | API RP 2016 — Guidelines and Procedures for Entering and Cleaning Petroleum Storage Tanks (latest ed.) |
| External | NFPA 350 — Guide for Safe Confined Space Entry and Work (current edition) |

Confined Space Entry Permit & Atmospheric Monitoring Log

[Insert CSE Permit form — ORG-NTD-000010-HSE-FRM-00012. Section 1: Permit data. Section 2: Atmospheric Monitoring Log (columns: Time, O₂%, LEL%, H₂S ppm, CO ppm, Tester initials). Section 3: Entrant headcount log.]

Rescue Plan & Equipment Checklist

[Insert site-specific rescue plan referencing RGLNG Emergency Response Procedure ORG-NTD-000010-HSE-PRC-00025. Equipment list: mechanical retrieval device, tripod, full-body harness, SCBA set, first-aid kit, stretcher.]

Alternative Entry Procedure (Short-Form)

For atmospheric-only hazards in spaces <4 feet deep that can be made safe by continuous forced-air ventilation alone, the Entry Supervisor may authorize entry using this short-form procedure. Continuous ventilation must maintain O₂ 19.5–23.5% and all toxic gases below 50% of their PEL/TLV. A full Permit is not required; document the entry on the Alternative Entry Log (ORG-NTD-000010-HSE-FRM-00013).

| Date: 4/9/2026Error! Use the Home tab to apply DateOnCover to the text that you want to appear here. | Error! Reference source not found. |  |
| --- | --- | --- |
| Rev.: XXXA | 000-NTD-000-xxx-xxx-000xx000-NTD-000-xxx-xxx-000xx |  |

| Date: 4/9/2026Error! Use the Home tab to apply DateOnCover to the text that you want to appear here. | Error! Use the Home tab to apply Title Cover Page to the text that you want to appear here. |  |
| --- | --- | --- |
| Rev.: XXXA | 000-NTD-000-xxx-xxx-000xx |  |